VRRP (Virtual Router Redundancy Protocol)

The Problem VRRP Solves

In a typical network, hosts use a default gateway to reach external networks. Example:

Client → Default Gateway → Internet

If that gateway router fails, all clients lose connectivity even if another router exists on the network.

Without VRRP you would need to:

Manually change the gateway on every host or wait for routing protocols to reconverge. Both are slow and operationally painful. VRRP solves this by creating a highly available virtual gateway.

Core Idea of VRRP:

VRRP allows multiple routers/servers to share a Virtual IP address (VIP). Only one node actively owns the VIP at a time. Clients use the VIP as their gateway, not the real IP of a specific machine.

Example:

Real IPs

Router1: 192.168.1.2

Router2: 192.168.1.3

Virtual IP (VIP)

192.168.1.1

Clients configure:

default gateway = 192.168.1.1

Internally:

Router1 (MASTER) → owns 192.168.1.1

Router2 (BACKUP) → waiting

If Router1 fails:

Router2 becomes MASTER → Router2 takes 192.168.1.1

Clients do not notice the change.

VRRP Terminology

- Virtual Router:

VRID 10

VIP: 192.168.1.1

Members:

node1

node2

A virtual router is the logical group of nodes sharing a VIP. Example:

- VRID (Virtual Router ID)

A number identifying a VRRP group.

range: 1 - 255 → Example: VRID = 51

Every router participating in the same virtual router must use the same VRID.

- Virtual IP (VIP)

The shared IP address used by clients.

Only the MASTER node binds this IP to its interface.

virtual_ipaddress {

192.168.1.1

}

Example:

- MASTER

The router currently responsible for:

- Holding the VIP

- Responding to ARP

- Forwarding traffic

- BACKUP

Routers that:

- Monitor the MASTER

- Wait for failure

- Take over if needed

How VRRP Failover Works:

VRRP nodes send advertisements periodically. Default is every 1 second. These advertisements say: "I am the MASTER". BACKUP nodes monitor these packets. If the advertisements stop, MASTER considered dead. Then election happens.

Election Algorithm

Each router has a priority value. It’s range is: 0 – 255. Default is 100. Higher priority wins.

Example:

- Node1 priority: 150

- Node2 priority: 100

Node1 becomes MASTER. If Node1 fails → Node2 takes over.

Virtual MAC Address

VRRP also defines a virtual MAC address.

Format: 00:00:5E:00:01:VRID

Example: VRID = 51 → MAC = 00:00:5E:00:01:33

Why this matters:

When MASTER changes, the MAC stays the same, preventing ARP instability. However, many implementations (including Keepalived) still send Gratuitous ARP to update switches.

In VRRP, the Virtual IP (VIP) is associated with a Virtual MAC address:

VIP → Virtual MAC (VRRP MAC)

Example:

VIP: 192.168.1.1

VRID: 51

Virtual MAC: 00:00:5E:00:01:33

Only the MASTER binds:

- the VIP

- the virtual MAC

to its network interface.

When MASTER changes:

Old MASTER goes down, New MASTER comes up → The VIP and its virtual MAC do NOT change.

This is fundamentally different from active/standby designs where each node has its own MAC.

In another words:

- The VRRP virtual MAC address belongs to the VRRP group, not to a specific machine.

- Only the MASTER actually uses (owns) that virtual MAC on its NIC.

- When the MASTER dies, the BACKUP that becomes MASTER:

- adds the same virtual IP (VIP)

- adds the same virtual MAC to its interface.

So:

Before failover:

- MASTER has VIP + virtual MAC

- BACKUP does not have VIP and virtual MAC

After failover:

- Old MASTER is dead

- New MASTER (previous BACKUP) now has the same VIP and the same virtual MAC.

Why this is good:

- Hosts do not need to relearn: 192.168.1.1 → some *new* MAC

- In theory, no ARP update should be needed because: VIP → same MAC

This is what the statement means by: “the MAC stays the same, preventing ARP instability”

So why is Gratuitous ARP still necessary?

Because ARP is not the only cache in the network. There are multiple layers of state involved.

Gratuitous ARP:

Gratuitous ARP is an unsolicited ARP reply that a host sends out either periodically or during IP address configuration. It announces or updates its own IP-to-MAC mapping in the network.

Why use Gratuitous ARP?

- IP Address Conflict Detection → When a device joins the network or changes its MAC address, it broadcasts a Gratuitous ARP to check if the IP address is already in use. If another device responds with the same IP, a conflict is detected.

- Updating Other Devices → It helps other hosts in the network update their ARP tables with the new MAC address for that IP, especially useful if a device’s MAC address has changed (like after network interface replacement or failover scenarios).

- Network Optimization → It can improve network efficiency by preemptively announcing the device’s MAC-IP association, reducing ARP requests later.

What changes during failover (even if MAC doesn’t)

Even though the VIP → MAC mapping is unchanged, the MAC → switch port mapping changes.

Before failover:

VIP → 00:00:5E:00:01:33

MAC 00:00:5E:00:01:33 → Switch port 1 (Old MASTER)

After failover

VIP → 00:00:5E:00:01:33   (same)

How switches learn MAC locations

Ethernet switches build a CAM table: MAC address → physical port

They learn this when they see a frame sourced from that MAC.

Problem:

After failover, the new MASTER might not send traffic immediately → Switch still believes: VRRP MAC → old port

Result:

- Packets are forwarded to the dead port

- Traffic is blackholed

Gratuitous ARP fixes this

A Gratuitous ARP (GARP) is an ARP packet sent without being requested.

Typical GARP:

MAC 00:00:5E:00:01:33 → Switch port 7 (New MASTER) → Switches must relearn where that MAC lives.

Sender IP: 192.168.1.1

Sender MAC: 00:00:5E:00:01:33

Target IP: 192.168.1.1

Target MAC: ff:ff:ff:ff:ff:ff

This does two things at once:

- Updates ARP caches on hosts

Even though MAC is the same, some hosts:

- flush ARP entries on link changes

- behave conservatively

- Forces switches to relearn MAC location

Because Frame source MAC = VRRP MAC → Switch sees 00:00:5E:00:01:33 is now on port 7. This is the real reason GARP is critical.

Why this matters more in real networks

- Switch CAM aging is slow

Default CAM aging is 300 seconds (5 minutes). Failover without GARP could cause minutes of outage.

- Multiple switches / L2 domains

In larger networks:

- upstream switches

- ToR switches

- VM bridges

- virtual switches (OVS, Linux bridge)

All need to relearn the MAC.

- Virtualization and cloud environments

Hypervisors and virtual switches are often more strict and may:

- drop frames

- delay learning

Keepalived sending GARP avoids subtle outages.

Keepalived sends multiple GARPs on transition to MASTER.

VRRP States:

A router in VRRP can be in 3 states.

- INIT → Initial state. Router not participating yet.

- MASTER → Router that:

- owns VIP

- sends advertisements

- forwards packets

- BACKUP → Router that:

- listens for advertisements

- waits for MASTER failure

Preemption

Preemption determines whether a higher priority node takes back MASTER role.

Example:

Node1 priority 150

Node2 priority 100

Scenario:

- Node1 fails

- Node2 becomes MASTER

- Node1 comes back

Two behaviors are possible.

- Preemption Enabled (default) → Node1 takes MASTER back immediately.

- Preemption Disabled → Node2 stays MASTER until it fails.

Keepalived config:

nopreempt → This prevents unnecessary role flapping.

VRRP Advertisements

VRRP packets are sent using:

Protocol: IP protocol 112

Multicast: 224.0.0.18

TTL: 255

Important security behavior:

Routers drop packets with TTL != 255 to prevent remote spoofing.

Why Keepalived Uses VRRP:

Keepalived implements VRRP in Linux. It is commonly used for:

  - Highly available load balancers

  - Highly available routers

  - Highly available services

VIP

10.0.0.10

|

---------------------------------

|                                          |

LB1 (MASTER)               LB2 (BACKUP)

|                                          |

------ backend servers ------

Example architecture:

If LB1 fails → LB2 takes VIP. Service remains available.

VRRP vs Other HA Approaches

- VRRP vs HSRP → HSRP is Cisco proprietary. VRRP is open standard (RFC 5798).

- VRRP vs CARP → CARP is used mainly in OpenBSD/pfSense. VRRP is the industry standard.

VRRP Authentication:

Older VRRP versions supported authentication, but it is deprecated in modern VRRP. Security usually relies on:

- network isolation

- TTL=255 rule

VRRPv2 vs VRRPv3:

- VRRPv2 → IPv4 only

- VRRPv3 → IPv4 + IPv6

Keepalived supports both.

Multiple VIPs

A VRRP instance can manage multiple IPs.

virtual_ipaddress {

192.168.1.10

192.168.1.11

192.168.1.12

}

Example:

Multiple VRRP Instances

Used for load distribution. Example:

VRRP1 → node1 master

VRRP2 → node2 master

This prevents idle backups.

Where VRRP Fails (Limitations)

Important real-world limitations.

Not Load Balancing → VRRP is failover only, not load balancing.

Layer 2 Requirement → VRRP requires nodes to be in the same L2 broadcast domain. It does not work across routed networks.

Failover Delay → Failover time depends on:

text

advertisement interval

priority

skew time

Typical failover:

text

~3 seconds

But with tuning it can be <1 second.